Classroom Hub Device Setup Guide

Use this guide when you are physically setting up the Classroom Hub device.

You are working on the local setup page:

http://rosehub.local/setup

This guide is only for the device setup work: power, hotspot, Wi-Fi, pairing, and downloading files. Use the Educator Dashboard Guide for accounts, classrooms, lessons, and sync management.

What You Need

•	
      Classroom Hub device

•	
      Wall charger or charged battery pack

•	
      Tablet, phone, or laptop

•	
      Internet Wi-Fi for setup and downloading lessons

•	
      Educator Dashboard account already created, or an educator ready to create one at rosepch.org

Picture to add: photo of the Classroom Hub device, charger, optional battery pack, and a tablet/laptop.

Step 1: Power On The Classroom Hub

The Classroom Hub can use wall power or a battery pack.

Wall Charger

1	
      Plug the power cable into the Classroom Hub.

2	
      Plug the charger into the wall.

3	
      Wait about 1 minute for the hub to start.

4	
      Look for the setup Wi-Fi network on your tablet, phone, or laptop.

Battery Pack

1	
      Make sure the battery pack is charged.

2	
      Plug the Classroom Hub power cable into the battery pack.

3	
      Turn on the battery pack if it has a power button.

4	
      Wait about 1 minute for the hub to start.

5	
      Keep the battery pack connected while teaching.

Picture to add: close-up photo of the Classroom Hub powered by a charger and another photo powered by a battery pack.

Step 2: Join The Hub Setup Hotspot

When the hub is not yet connected to internet Wi-Fi, it creates a temporary setup network.

The network name looks like:

Rose-Setup-XXXX

The setup hotspot password is:

rosehub1

1	
      Open Wi-Fi settings on your tablet, phone, or laptop.

2	
      Choose the Rose-Setup-XXXX network.

3	
      Enter the password rosehub1.

4	
      Wait for your device to connect.

Picture to add: screenshot of Wi-Fi settings showing a Rose-Setup-XXXX network.

Step 3: Open Classroom Hub Device Setup

After joining the setup hotspot, the setup page may open automatically.

If it does not open:

1	
      Open a browser.

2	
      Go to:

http://rosehub.local/setup

1	
      Follow the setup page.

Picture to add: screenshot of the first Classroom Hub Device Setup screen.

Step 4: Connect The Hub To Internet Wi-Fi

The hub needs internet during setup and when downloading new lessons.

1	
      On the setup page, choose the internet Wi-Fi network the hub should use.

2	
      Enter the Wi-Fi password.

3	
      Submit the form.

4	
      Wait about 1 minute.

5	
      Reconnect your tablet, phone, or laptop to the same internet Wi-Fi network.

6	
      Open:

http://rosehub.local/setup

If the hub cannot connect to internet Wi-Fi, it will return to the Rose-Setup-XXXX hotspot so you can try again.

Picture to add: screenshot of the Wi-Fi selection screen on the setup page.

Step 5: Show The Pairing Code

After the hub is online, the setup page shows a one-time pairing code.

This code is entered in the Educator Dashboard at rosepch.org.

1	
      Keep the setup page open.

2	
      Find the pairing code on the page.

3	
      Give the pairing code to the educator using the Educator Dashboard.

4	
      Wait on this page while the educator links the hub.

The setup page checks automatically. When the hub is linked, it will move to the next step.

Picture to add: screenshot of the pairing code on the hub setup page.

Step 6: Download Latest Files

After the hub is linked, the educator creates classrooms and lessons in the Educator Dashboard.

When lessons are ready for the hub:

1	
      Keep the Classroom Hub powered on.

2	
      Make sure it is connected to internet Wi-Fi.

3	
      On the setup page, choose Download latest files.

4	
      Wait until the page says the Student Library is ready.

Picture to add: screenshot of the download/latest files step.

Step 7: Open The Student Library

When setup is complete, students use the Student Library.

Student address:

http://rosehub.local/join

Students will need the classroom join code from the educator.

Picture to add: screenshot of the Student Library join page.

Troubleshooting

I Cannot See Rose-Setup-XXXX

•	
      Wait 1 minute after plugging in the hub.

•	
      Move closer to the hub.

•	
      Turn Wi-Fi off and on again on your tablet, phone, or laptop.

•	
      Restart the hub by unplugging power, waiting 10 seconds, and plugging it back in.

The Setup Page Does Not Open

•	
      Make sure you are connected to the Rose-Setup-XXXX Wi-Fi network first.

•	
      Open a browser and type http://rosehub.local/setup.

•	
      If that does not work, try again after restarting the hub.

The Hub Will Not Connect To Internet Wi-Fi

•	
      Check the Wi-Fi password.

•	
      Choose the correct network name.

•	
      Move the hub closer to the router.

•	
      Try again from the setup page.

Some Wi-Fi networks that require a separate agreement page may not work with the hub.

The Hub Does Not Show As Linked

•	
      Make sure the hub has internet.

•	
      Make sure the educator entered the pairing code in the Educator Dashboard.

•	
      Keep the setup page open for a few seconds; it checks automatically.

•	
      If needed, choose Check status.

Students Cannot Open The Student Library

•	
      Make sure the Classroom Hub is powered on.

•	
      Make sure students are connected to the hub Wi-Fi network.

•	
      Make sure students open http://rosehub.local/join.

•	
      Make sure students have the correct classroom join code.

Device Setup Checklist

•	
      ☐ Classroom Hub is powered on

•	
      ☐ Tablet/phone/laptop joined the setup hotspot

•	
      ☐ Hub connected to internet Wi-Fi

•	
      ☐ Pairing code was shown

•	
      ☐ Hub was linked in the Educator Dashboard

•	
      ☐ Latest files were downloaded

•	
      ☐ Student Library opened successfully